|  | 17 July 98 |
| --- | --- |
| FOUNDATION FOR INTELLIGENT PHYSICAL AGENTS | Dublin |
| Agent Management Technical Committee |  |

FIPA ’98 Draft Specification: Part 10

Agent Security Management

Version : FIPA8714e

Revision 0.5

Editors: Wayne Jansen, Kari Koivuniemi, Michael Montag, Jorge Núñez-Suárez, Alexander Yip

Past Editor: Marius Tesselaar

© 1998 FIPA - Foundation for Intelligent Physical Agents

Geneva, Switzerland

Contents

1.	Problem Statement	4

2.	Scope	5

3.	Normative References	7

4.	Security Management	7

4.1	Security Related Platform Interfaces	7

4.1.1	The Directory Facilitator (DF)	7

4.1.2	The Agent Management System (AMS)	7

4.2	Certificate Management Interfaces	8

4.3	Message Encapsulation Interfaces	8

4.3.1	Agent Level Protection	9

4.3.2	Transport Level Protection	9

5.	Security Management Ontology	11

5.1	Agent Security Management Actions	11

5.2	Agent Security Management Object Descriptions	11

5.3	FIPA-Security-Management Parameters	11

5.4	FIPA-Security-Management Agents	11

5.4.1	fipa-man-df-agent-description for the fipa-security-man-agent type	11

5.5	FIPA-Security-Management Services	12

5.5.1	fipa-man-service-description for the fipa-certification-authority-service type	12

5.5.2	fipa-man-service-description for the fipa-certificate-repository-service type	12

5.5.3	fipa-man-service-description for the fipa-security-enhanced-agent-service type	13

6.	Agent Security Management Operations	14

6.1	generate_certificate	14

6.2	revoke_certificate	15

6.3	provide_certificate	16

6.4	provide_crl	17

6.5	receive	18

TO DO LIST

|  | Who | When |
| --- | --- | --- |
| - review document | members TC1 | NC USA(Oct 98) |
| - provide an example scenario of security features for a simple agent life cycle | members of security subgroup of TC1 (TC1-SG) | NC |
| - extend / refine the ontology | members TC1-SG | NC |
| - refine the normative references | members TC1-SG | NC |
| - develop concrete examples for the use of transport level security, e.g. CORBA security services or SSL. | members TC1 / TC1-SG | NC |
| - provide agent security management object descriptions | members TC1-SG | NC |
| - investigate the need for helper security agents and specify the according operations | members TC1 / TC1-SG | NC |
| - mobility: specify security ontology including operations for mobility | members TC1 / TC1-SG | NC/Korea (Jan 99) |
| - provide exception handling descriptions for the different operations | members TC1-SG | NC |

# Problem Statement

Security risks exist in various domains including agent management, life cycle management, registration, agent platforms, agent-agent interaction, user-agent interaction and mobility.  Some of these security risks have been identified and will be handled by existing counter measures that are well known and suitable for inclusion into this document.

However, there still may be agent specific security risks that so far have not been identified.  Particularly, the agent specific paradigm of non-deterministic intelligent autonomous collaboration probably gives rise to new security risks comparable to those known from real world social societies.  At least for Multi Agent Systems, common security measures e.g. for denial of service attacks do currently not exist.

The deployment of Agent Technology implies various security risks that need to be addressed.  Existing security solutions have to be identified and investigated in order to assess their suitability and applicability.  Existing solutions will be facilitated when appropriate.

The following six security threats currently exist which all apply to Agents and Multi Agent Systems:

- Disclosure (Eavesdropping)

- Alteration (including Mutilation and Corruption)

- Copy and Replay

- Denial of Service (including Resource Exhaustion)

- Repudiation

- Spoofing (Masquerading)

The following examples demonstrate the threats in Agent Systems.

Disclosure:

An entity eavesdrops the interaction among agents extracting information on the goals, plans, capabilities, etc.  of the these agents.

Alteration:

An Agent Communication Channel modifies content of messages.

Copy and Replay:

The message from an Agent is illegally copied and sent with new directions to certain resources.

Denial of Service:

A Directory Facilitator Agent refuses the registration of another agent and hence prevents it from achieving its goals.

Repudiation:

A commitment between two agent as the result of a contract negotiation is later ignored by one of them denying the negotiation has ever taken place and its part of the commitment will not be fulfilled.

Spoofing and Masquerading:

An Agent registers as a Directory Facilitator Agent and therefore receives information form other registering agents.

# Scope

FIPA 98 Security Management addresses mutual Agent security issues for Agent to Agent interaction based upon the following assumptions and guiding principles:

- This specification does not mandate the use of security features.  Instead, it mandates how agents and agent platforms may interoperate in a secure fashion, if security desired.

- Public key infrastructures (PKI), based on X.509 public key certificates and certificate revocation lists, will exist in the future and will be wrapped as a certificate server for access by agents and agent platforms.  This specification defines management operations for access to such wrappers.

- This specification does not define how a PKI is established nor how initial public key pairs and certificates are established for agents and agent platforms.  However, the security capabilities defined in this specification rely on their existence.

- This specification allows for security management to be implemented at both the agent level, through direct encapsulation/decapsulation of ACL messages by cooperating agents, and at message transport layer, through the use of security services available from a shared transport protocol at the agent platform.

- At the agent level, this specification relies on an asynchronous messaging model of communications, as opposed to a session-oriented model.  All information regarding the protection mechanisms employed to encapsulate a given message is provided with the message.  At the transport level, the protection mechanisms may be either message or session-oriented.  An agent is free to choose any appropriate transport level mechanism available at the agent platform where it is situated.

- Payment protocols (e.g., SET) or micro payment mechanisms are not addressed in this specification.

- Mobile agent security is not currently addressed by this specification.  This topic will be addressed in future drafts.

- For the security capabilities described by this specification to work properly, the AMS, ACC, and DF components that form a platform must communicate securely through some means outside the scope of the specification.

- The DF is a useful way of discovering capabilities of a registered agent.  By extending the information registered to include security-related information, the DF can provide additional complimentary support for the security functions described in this specification.

- The platform profile can become a standard way for an agent to discover the security transport mechanism supported at the agent platform, if its contents are extended accordingly and a means of inquiry is provided.

The following security capabilities will be further explored and specified in FIPA 98 as a normative security framework:

- Authentication

- Integrity

- Confidentiality

- Security Management

- Including:

- Certificate creation / deletion

- Certificate repository retrieval

- Certificate revocation

FIPA 98 Security Management does not provide specifications supporting:

- Security measures for regular system administration

- Security measures for non-agent software that is corrupted and poses a security threat to the agent system

- Security measures for non-agent related security issues, for example user identification to operating systems

FIPA 98 Security Management will reuse and refer to existing security standards and solutions whenever possible, such as the PKIX standard from the IETF [3].

# Normative References

[1]	FIPA 97 Parts 1-3

[2]	ITU X.509 v3

[3]	PKIX Standard IETF, URL <http://www.ietf.org/internet-drafts/draft-ietf-pkix-ipki3cmp-08.txt>

[4}	Cryptographic Message Syntax Standard, Public Key Cryptography Standards (PKCS) #7, RSA Laboratories, November 1993, URL <http://www.rsa.com/rsalabs/pubs/PKCS/html/pkcs-7.html>

# Security Management

## Security Related Platform Interfaces

### The Directory Facilitator (DF)

Several parameters defined in fipa-man-df-agent-descriptions may be usable in determining the security context of a registered agent.  They include the :interaction-protocols, :agent-type, :agent-services, and :ownership parameters.  For example, an agent may wish to use the security services of a CA.  The :agent-services parameter indicates whether CA services are supported by any agent registered by that DF.  When a search operation is used to discover an agent by a particular entity, the :agent-services parameter can be used either to constrain the search to an agent which supports CA services, or to select from the results of a search which was not constrained by this parameter.  This facility, of course, requires that agents provide the search parameters of interest when they register with the DF.

This specification extends the set of defined parameters for DF register entries to provide more support for security operations.  The new parameter security-context is defined which may contain the following parts:

- :agent-certificate

- :owner-certificate

- :security-encapsulation-method

The :agent-certificate and :owner-certificate parts respectively convey one or more  public key certificates of an agent and of the agent’s owner.  The :security-encapsulation-method conveys the various methods of encapsulation supported directly by the agent.  Multiple certificates apply in situations where an agent or an agent’s owner has established public key pairs for different policies, functions, or domains.  With the addition of the first two parts, the DF essentially becomes a default repository for agent certificates.  Since public key certificates are signed objects, there is no requirement on the DF to protect this information beyond that of any other parameter.  These optional parameters, if present, are necessary for security encapsulation to occur.

Note that there is always one DF for each agent platform, referred to as the default DF.  A DF may register with other DFs to represent an arbitrary network of relationships.  While the DF can be considered a form of certificate repository, it is not a replacement for repositories that may be established as part of a general, public key certificate infrastructure.  The latter, for example, would contain certification revocation lists needed to verify that a particular certificate has not been revoked before its expiration date.  Both the ACC and AMS can register with a DF.  At a minimum, the AMS must register with the default DF of the platform.

### The Agent Management System (AMS)

The AMS is responsible for managing the essential activities of a agent platform, including creation/deletion of agents, registration of agents at the platform, and control over access to and use of the ACC.   Moreover, there is only one AMS for each agent platform.  The AMS for the platform on which an agent is created is called the home agent platform, responsible for vouching for an agent’s identity.  If the agent platform is in any way considered to be trusted, that trust must begin with the AMS.   Therefore, it is reasonable to consider that an AMS may possess a public key pair and associated certificate that among other things provides the basis for inter-platform security from the message transport service.

The basic capabilities of the platform are described within the platform profile.  This specification extends the platform profile with an additional parameter, :transport-encapsulation-protocols.  The :transport-encapsulation-protocols indicates the various methods of encapsulation supported directly by the platform message transport security protocols.  In order to make use of this information, a new command is needed to query the AMS (see FIPA98 Part 1 for details). Besides the new command, this specifications requires that each agent registering with the AMS is provided the platform profile information in the response.

## Certificate Management Interfaces

Agent security services described in this specification rely on the use of public key cryptography for encryption and signature, and digital certificates for binding the identities of agents and agent platforms with their associated public key material.  Digital certificates are normally signed by a certification authority (CA) and maintained elsewhere in a certificate repository (CR).  The certificate repository also maintains a list of revoked certificates (i.e., a certificate revocation list).  Collections of certification authorities and certificate repositories operating under a common scheme make up a public key infrastructure (PKI)

An agent or agent platform can request the creation of a certificate from a certification authority.  In order to supply the required information for certificate creation, a public and private key pair must be generated.  The specification assumes that the agent or agent platform is able to obtain a key pair and only requires access to a security management agent for certificate creation.

Once a key pair is obtained, the agent or agent platform passes its identity and public key to the CA security management agent.  The CA validates the request, generates the certificate, issues the certificate to the requestor, and registers the certificate in an associated certificate repository.  Validation may require off-line communication between owner of the requesting entity and the operator of the certification authority.  The generated certificate binds the identity of the requesting agent with its public key.  The certificate also identifies the certification authority that signed the certificate and related policy information such as the security policy identifier, validation period, etc.  An agent may require more than one certificate to support different domains or applications, or for specific functions within a domain, or for other reasons.

The exact details of a security encapsulation differ based on the threats being countered, the syntax of the security token, the various cryptographic algorithms being applied, the sizes of keys, etc.  The intent of this specification is to allow complete freedom on the method of encapsulation used.

## Message Encapsulation Interfaces

An agent has two methods for protecting ACL communication with other agents:

- Use a new management action, the receive action, to convey a message the agent encapsulates itself directly

- Use the :envelope parameter at the ACL message level in conjunction with a secure transport protocol mechanism

Both modes of protection may require the communicating entities, whether an agent or agent platform, to possess valid public key certificates. The following sections provide examples of how the encapsulation can be done, but it is not prescriptive and the specific details are left open to implementor.

### Agent Level Protection

One way to send an ACL message confidentially to another agent, is that the initiating agent first obtains the digital certificate of the intended recipient agent.  This allows the initiating agent to encrypt the message, typically by generating a random secret key, encrypting the message with the secret key, then encrypting the secret key with the recipient’s public key.  Once the message is encrypted, it is conveyed to the recipient.  The recipient agent can then decrypt the message using its private key to unlock the secret key, and then the secret key to unlock the message.

Message integrity is obtained in a manner somewhat similar to message confidentiality.  Rather than using the public key of the recipient of the message, the initiating agent must compute a message digest over the message body and digitally sign it using its own private key.  The recipient agent can then confirm the signature by securely obtaining the initiating agent’s certificate and utilizing the public key within the certificate to verify the message digest.  Using digital signature for message integrity implicitly provides message source authentication and possibly non-repudiation (depending on whether the generation of the private key was under the control of the signer).

In order to allow for flexibility on the method of encapsulation used by an agent, a variety of different security tokens and associated algorithms can be selected.  This specification relies on the observation that the syntax of most, if not all, present-day message security encapsulation formats is expressed in terms of an abstract syntax notation.  One such standard representation for message encapsulation is the Cryptographic Message Syntax Standard [4], which defines several content types, three of which correspond to the security services discussed so far:

- Signed-data content type  message integrity and source authentication

- Enveloped-data content type  message confidentiality

- Signed-and-enveloped-data content type  message confidentiality, integrity, and source authenticaiton

One other benefit of using ASN.1 syntax definitions is that they can be officially registered, providing an unambiguous value for the :syntax-id parameter, which is a new ACL message parameter used to convey the syntax of the security token.

### Transport Level Protection

Transport level protection relies on the :envelope parameter to indicate the security services the transport mechanism will apply to a message.  The entire ACL message, including the :envelope parameter, is treated as the payload and processed (e.g., encrypted, signed, etc.) accordingly.  While the security services are indicated by the agent, the responsibility for encapsulating the message lies with the agent system transport mechanism.

The format of the :envelope parameter information is a list of keyword/value pairs.  The keyword “security” identifies transport level encapsulation.  The keyword is followed by one of the following values: “confidentiality”, “integrity”, “confidentiality-authentication”, “integrity-authentication”, and “confidentiality-integrity-authentication”.  The mapping of these values to specific transport level security services depends on the particular protocols supported by the agent platform.  The table below identifies typical interpretations for common transport protocol mechanisms. This list may be expanded to include other security requirements, such as payment mechanism.

Service Mappings for the :envelope Parameter

| Value of the security keyword | Description |
| --- | --- |
| confidentiality | The message is encrypted for the receiver. An eavesdropper won’t be able to view or derive the original plaintext. |
| integrity-authentication | A one-way hash of the message (i.e., message digest) is calculated and signed on behalf of the sender and sent along with the message to the receiver. The recipient is then able to verify the sender’s identity and whether the message has been tampered with. |
| confidentiality-integrity-authentication | A message digest is calculated for the message. The digest is signed on behalf of the sender, then encrypted for the receiver. |

# Security Management Ontology

## Agent Security Management Actions

AgentSecurityAction =

"(" "generate-certificate fipa-security-description ")"

|"(" "revoke-certificate fipa-security-description ")"

|"(" "provide-certificate fipa-security-description ")"

|"(" "provide-crl fipa-security-description ")"

|"(" "receive fipa-security-description ")"

## Agent Security Management Object Descriptions

<<

TO BE DONE

>>

x509v3-certificate-description="ref: ITU X.509 v3 [2]"

x509v3-crl-description=”ref: ITU X.509 v3 [2]”

fipa-certificate="(" "x509v3-certificate" x509v3-certificate-description")"

fipa-crl=”(“ “x509v3-crl” x509v3-crl-description”)”

fipa-credentials=

token=

syntax-id=

## FIPA-Security-Management Parameters

| Parameter | Description |
| --- | --- |
| fipa-certificate | The ITU X.509 certificate that is used to authenticate agents. |
| fipa-credentials | The information that is needed by the CA to generate a certificate. |
| token | A selectable security message data structure (e.g., PKCS #7) that contains information about the security encapsulation of an ACL message, including the identifiers of algorithms used to sign and encrypt the message, message encryption key encrypted with the public key of the receiver, and a message digest signed with the private key of the sender, as required. |
| syntax-id | The object identifier of the registered abstract syntax associated with the security token for an encapsulated message. |
| fipa-crl | The ITU X.509 certificate revocation list that is used to determine whether a certificate has been revoked and no longer valid |

## FIPA-Security-Management Agents

### fipa-man-df-agent-description for the fipa-security-man-agent type

| Parameter | Description |
| --- | --- |
| :agent-name | SecurityManagementAgent |
| :agent-type | fipa-security-man-agent |
| :agent-services | fipa-certification-authority-service fipa-certificate-repository-service fipa-security-enhanced-agent-service |
| :interaction-protocols |  |
| :ontology |  |
| :agent-address |  |
| :ownership |  |
| :df-state |  |

## FIPA-Security-Management Services

The following presents a list of service types available in the fipa-security-management ontology.  An agent of type fipa-security-management-agent may register one or more of these services with a DF, as specified by FIPA 97 part 1 sections 9.3.1 and 9.3.3, and FIPA 97 part 3 section 6.2.1.1.1.

### fipa-man-service-description for the fipa-certification-authority-service type

#### Overview

| Parameter | Description |
| --- | --- |
| :service-name | CertificationAuthority |
| :service-type | fipa-certification-service |
| :service-ontology | fipa-certification |
| :fixed-properties |  |
| :negotiable-properties |  |
| :communication-properties |  |

#### Actions

The following actions, with their parameters and possible results, are those that an agent must be able to perform when supporting this service.

| actions | input | output |
| --- | --- | --- |
| revoke-certificate |  |  |
| generate-certificate | fipa-credentials | fipa-certificate |

### fipa-man-service-description for the fipa-certificate-repository-service type

#### Overview

| Parameter | Description |
| --- | --- |
| :service-name | CertificateRepository |
| :service-type | fipa-certificate-repository-service |
| :service-ontology | fipa-certificate-repository |
| :fixed-properties |  |
| :negotiable-properties |  |
| :communication-properties |  |

#### Actions

The following actions, with their parameters and possible results, are those that an agent must be able to perform when supporting this service.

| actions | input | output |
| --- | --- | --- |
| provide-crl |  |  |
| provide-certificate | fipa-certificate |  |

### fipa-man-service-description for the fipa-security-enhanced-agent-service type

#### Overview

| Parameter | Description |
| --- | --- |
| :service-name | SecurityEnhancedAgent |
| :service-type | fipa-encapsulation-service |
| :service-ontology | fipa-encapsulation |
| :fixed-properties |  |
| :negotiable-properties |  |
| :communication-properties |  |

#### Actions

The following actions, with their parameters and possible results, are those that an agent must be able to perform when supporting this service.

| actions | input | output |
| --- | --- | --- |
| receive | token syntax-id |  |

# Agent Security Management Operations

<<

TO BE DONE:

- refinement of :content

- refine actions and parameters supplied

- add exception handling for the operations

- consistency check

>>

## generate_certificate

| Supported by | CA agent |  |
| --- | --- | --- |
| Description | The CA agent generates a certificate upon request. It checks the credentials of the requesting agent before it generates the certificate. The CA agent sends the certificate to the requesting agent. |  |
| Content | fipa-security-description |  |
| FIPA Protocol | FIPA-request (see FIPA 97 Part 2) |  |
| Example | (request : sender an-agent : receiver an-agent : content ( :fipa-certificate :action generate-certificate ( :fipa-credentials ) ) : language SL0 : protocol fipa-request : ontology fipa-security ..... ) |  |
| Refuse Reasons |  |  |
| Failure Reasons |  |  |

## revoke_certificate

| Supported by | CA Agent |  |
| --- | --- | --- |
| Description | The CA Agent revokes the certificate upon request by the agent or a recognized organizational authority. |  |
| Content | fipa-security-description |  |
| FIPA Protocol | FIPA-request (see FIPA97 Part 2) |  |
| Example | (request : sender an-agent : receiver an-agent : content ( :action revoke-certificate ( :fipa-certificate ) ) : language SL0 : protocol fipa-request : ontology fipa-security ..... ) |  |
| Refuse Reasons |  |  |
| Failure Reasons |  |  |

## provide_certificate

| Supported by | CR Agent |  |
| --- | --- | --- |
| Description | The CR agent provides a list of certificates associated with a particular agent upon request by another agent. |  |
| Content | fipa-security-description |  |
| FIPA Protocol | FIPA-query (see FIPA97 Part 2) |  |
| Example | (query_ref : sender an-agent : receiver an-agent : content ( :fipa-certificate ) : language SL1 : protocol fipa-request : ontology fipa-security ..... ) |  |
| Refuse Reasons |  |  |
| Failure Reasons |  |  |

## provide_crl

| Supported by | CR Agent |  |
| --- | --- | --- |
| Description | The CR agent provides a certificate revocation list for a certification authority of an agent, upon request by another agent. |  |
| Content | fipa-security-description |  |
| FIPA Protocol | FIPA-query (see FIPA97 Part 2) |  |
| Example | (query_ref : sender an-agent : receiver an-agent : content ( :fipa-crl ) : language SL1 : protocol fipa-request : ontology fipa-security ..... ) |  |
| Refuse Reasons |  |  |
| Failure Reasons |  |  |

## receive

| Supported by | Security Enhanced Agent |  |
| --- | --- | --- |
| Description | The receiving agent decapsulates the received message according to the information contained within the :security-token parameter. The decapsulation process includes decryption of the contained ACL message and verification of the signature of the message source, if the sending agent applied the reverse of these processes during security encapsulation. |  |
| Content | fipa-security-description |  |
| FIPA Protocol | FIPA-request (see FIPA97 Part 2) |  |
| Example | (request : sender an-agent : receiver another-agent : content ( : action another-agent (receive (: protected-content token) (: token-type syntax-id) ) ) : language SL1 : protocol fipa-request : ontology fipa-security ..... ) |  |
| Refuse Reasons |  |  |
| Failure Reasons |  |  |