| 11th Meeting | OPIMA | Specification |
| --- | --- | --- |
| Turin | Open Platform Initiative for Multimedia Access | Version 1.1 |
| 00/06/27 |  |  |

OPIMA Specification

Version 1.1

Disclaimer

OPIMA reserves the right to modify the contents of this document according to its procedures for technical work and without notice. OPIMA makes no claim as to operational integrity of any implementation derived from this document. This document is provided as is with no warranties whatsoever.

This OPIMA specification has been developed and published in the framework of the International Electrotechnical Commission's (IEC) Industry Technical Agreement (ITA) programme.

Table of Contents

1	Introduction (Informative)	5

2	The OPIMA Architecture (Informative)	6

2.1	Credential mechanisms in the OPIMA environment	7

2.2	Required Trusted Institutions	7

2.2.1	Compartment ID issuance authority	8

2.2.2	Credential issuance authorities	8

2.2.3	IPMP Systems ID issuance authorities	8

2.2.4	OPIMA Peer ID issuance authorities	8

2.2.5	Encryption, Signature and Watermarking ID issuance authorities	8

2.3	Back-end Infrastructure	8

2.4	Protocols	8

3	The OPIMA Architecture (Normative)	10

3.1	OPIMA Protocols	10

3.1.1	First layer: Secure Authenticated Channel	10

3.1.2	Second layer: OPIMA Common Message Protocol	10

3.1.2.1	Open IPMP System download Message	10

3.1.2.2	IPMP System code Messages	10

3.1.2.3	Close of the OPIMA download protocol	11

3.1.2.4	Message IDs	11

3.2	Credential Formats	11

3.3	OPIMA Peer	11

3.3.1	OPIMA Virtual Machine	12

3.3.2	IPMPs	12

3.3.3	Application Services API	12

3.3.3.1	useContent	12

3.3.3.2	getIpmpSystem	14

3.3.3.3	queryOVM	14

3.3.3.4	sendMessageToIPMP	15

3.3.3.5	notifyEvent	16

3.3.3.6	Pseudo-code example for using the Application Services API	16

3.3.4	IPMP Services API	17

3.3.4.1	User Interface methods	17

3.3.4.1.1	sendMessageToUser	17

3.3.4.1.2	receiveMessageFromUser	18

3.3.4.2	Secure Storage Interface	18

3.3.4.2.1	secureStoreData	18

3.3.4.2.2	secureRetrieveData	19

3.3.4.2.3	secureDeleteData	19

3.3.4.3	Encryption and Decryption Engines	20

3.3.4.3.1	queryEncryptionAlgorithms	20

3.3.4.3.2	encrypt	20

3.3.4.3.3	initEncryption	20

3.3.4.3.4	updateEncryptionKeys	21

3.3.4.3.5	stopEncryption	21

3.3.4.3.6	decrypt	21

3.3.4.3.7	initDecryption	22

3.3.4.3.8	updateDecryptionKeys	22

3.3.4.3.9	stopDecryption	22

3.3.4.4	Signature Engines	22

3.3.4.4.1	querySignatureAlgorithms	23

3.3.4.4.2	verifySignature	23

3.3.4.4.3	verifyContentSignature	23

3.3.4.4.4	generateSignature	24

3.3.4.4.5	generateContentSignature	24

3.3.4.5	Watermarking Engines	25

3.3.4.5.1	queryWatermarkAlgorithms	25

3.3.4.5.2	extractWatermark	25

3.3.4.5.3	stopWatermarkExtraction	25

3.3.4.5.4	newWatermark	26

3.3.4.5.5	insertWatermark	26

3.3.4.5.6	stopWatermarkInsertion	26

3.3.4.6	Smart Cards	27

3.3.4.6.1	addCTListener	27

3.3.4.6.2	removeCTListener	27

3.3.4.6.3	getSlotId	28

3.3.4.6.4	isCardPresent	28

3.3.4.6.5	openSlotChannel	28

3.3.4.6.6	closeSlotChannel	29

3.3.4.6.7	getATR	29

3.3.4.6.8	reset	29

3.3.4.6.9	sendAPDU	30

3.3.4.6.10	cardInserted	30

3.3.4.6.11	cardRemoved	30

3.3.4.7	Abstract Access to Content	30

3.3.4.7.1	installCallbackContentAccess	30

3.3.4.7.2	abstractContentAccess	31

3.3.4.7.3	replyToContentAccess	31

3.3.4.8	Abstract Access to Rules	32

3.3.4.8.1	obtainUserRules	32

3.3.4.8.2	obtainContentRules	32

3.3.4.8.3	newRules	33

3.3.4.8.4	updateContentRules	33

3.3.4.9	Abstract Access to OPIMA Peers	34

3.3.4.9.1	openConnection	34

3.3.4.9.2	closeConnection	34

3.3.4.9.3	addConnectionListener	35

3.3.4.9.4	sendMessage	35

3.3.4.9.5	newConnection	35

3.3.4.9.6	receiveMessageFromPeer	36

3.3.4.10	Abstract Access to Applications	36

3.3.4.10.1	installCallbackApplication	36

3.3.4.10.2	replyMessage	36

3.3.4.10.3	receiveMessageFromApplication	37

3.3.4.11	Life-cycle Control	37

3.3.4.11.1	initialize	37

3.3.4.11.2	terminate	37

3.3.4.11.3	remove	38

3.3.4.11.4	update	38

3.3.4.12	Locale interface	38

3.3.4.12.1	getTime	38

3.3.4.12.2	getCountry	38

3.3.4.12.3	getLanguage	38

4	IDL definition of the OPIMA APIs (Normative)	40

5	Annexes (Informative)	44

5.1	Acronyms	44

5.2	Definitions	44

# Introduction (Informative)

This specification has been produced by OPIMA (Open Platform Initiative for Multimedia Access), an initiative in the Industry Technical Agreement (ITA) program of the International Electrotechnical Commission (IEC).

OPIMA has been established for the purpose of enabling a framework where content and service providers have the ability to extend the reach of their prospective customers and consumers have the ability to access a wide variety of content and service providers in a context of multiple content protection systems.

This specification describes the elements of The OPIMA platform. This platform is targeted at providing value-chain participants with the ability to acquire, supply, process and consume multimedia services on a global basis in accordance with the rights associated with these services. This specification specifically addresses intellectual property management and protection.

This specification presents an architecture and a description of the functions required to implement an OPIMA-compliant system. Further it presents security protocols and a description of Application Programming Interfaces (APIs) and functional behaviours that enable interoperability.

This specification contains four chapters and two Annexes. Chapter 1 is this Introduction. Chapter 2 provides a description of the OPIMA architecture. Chapter 3 provides the normative OPIMA specification with definitions of the APIs and a description of their behaviours. Chapter 3 also provides the OPIMA reference model. Chapter 4 is also normative and contains the IDL definition of the OPIMA APIs. Annexes give acronyms and definitions.

This version 1.1 of OPIMA specification is released for use by interested parties. OPIMA makes no claim as to operational integrity of any implementation derived from this document. This document is provided as is with no warranties whatsoever.

OPIMA is keen on receiving comments from implementers and will consider any comment received. Implementers are advised to consult the OPIMA web site

http://www.cselt.it/opima/

and subscribe to

Opima-ver@kim.cselt.it

To subscribe: send a message to majordomo@kim.cselt.it with “subscribe opima-ver” in the body of the message.

At a later date OPIMA may specify conformance-testing methodologies. However, actual conformance tests will not be carried out by OPIMA. This specification may be updated in the future without notice.

# The OPIMA Architecture (Informative)

This chapter contains an architectural overview of the OPIMA environment. It presents a framework for inter-operation between OPIMA compliant devices, called OPIMA Peers. The framework is designed so that code can be executed in the user environment without possibility to be tampered with by the environment. Therefore the code and associated content can only be used according to the rules associated with the content. The OPIMA Peer can also be used to interface with the back-end infrastructure.

The OPIMA specification is device and content independent. Content includes all multimedia types and executables. The specification is independent of all digital content processing devices and content types. The term Rules refers to information that stipulates how content may be used on a given device; specifically, Rules determine how business models are established. An IPMP (Intellectual Property Management and Protection) system controls access and use of the content by enforcing the rules associated with it. Conditional access systems are particular examples of IPMP systems.

Protected Content consists of:

- a content set, which may consist of multiple media types;

- an IPMP system set, which may consist of multiple IPMP systems;

- a rules set that applies under the given IPMP system.

OPIMA acknowledges that there is a world of proprietary domains that have their own governance structures that are defined by a set of IPMP systems. An example is provided by traditional Conditional Access systems. OPIMA may be used to enable interoperability among such system components. OPIMA may also be used as a bridge between proprietary domains.

OPIMA provides tools to exchange a set of authenticated identifiers, called OPIMA Credentials to enable Protected Content to flow within and between compartments. When OPIMA peers have an on-line connection, the OPIMA Credentials can be exchanged prior to the delivery of the Protected Content. OPIMA Credentials and IPMP Systems can be combined with the Protected Content when the delivery infrastructure does not support on-line connections (e.g. storage media, broadcast media). The OPIMA Credentials contain necessary information that may be used to enable Protected Content flow between compartments. For a given instance of Protected Content that is intended for consumption in two compartments, a Broadcast conditional access compartment and an Internet music delivery compartment, OPIMA Credentials from both compartments are associated with the Protected Content.

OPIMA enables generic interoperability between different applications, devices and IPMP systems belonging to different compartments. A compartment is a class of OPIMA enabled devices that share some common elements in their IPMP interfaces and/or architectural components. For example, DVB can be considered as a compartment, which in turn contains other compartments defined by specific IPMP system. Content does not necessarily flow between all compartments, however, OPIMA provides a schema for facilitating content flow between compartments. Compartments can be hierarchical. That is, a compartment can contain sub-compartments. OPIMA may also be used to provide interoperability within compartments even though in this case interoperability may be achieved by different means.

The OPIMA architecture is peer-to-peer. The core OPIMA element is a peer called the OPIMA Virtual Machine (OVM). OPIMA provides protocols and infrastructure components that enable secure (trusted) inter-operation amongst these elements. The peer-to-peer interaction based approach allows the efficient implementation of traditional client-server configurations.

OPIMA protocols are those protocols that enable the establishment of Secure Authenticated Channels and the downloading of IPMP Systems into the OVM.  Proprietary protocols may be used for the same purpose between peers within a compartment. In this case OPIMA protocols may also be used. OPIMA protocols must be used to download IPMP Systems between different compartments. Communication between IPMP Systems installed on OPIMA peers is effected using proprietary protocols, possibly on top of OPIMA protocols.

The aggregate of secure (trusted) interactions between OVMs represents the OPIMA environment. However, OVMs need not be continuously connected to the rest of the OPIMA environment. An OVM is able to function in a connected (one and two-way) and/or disconnected manner.

## Credential mechanisms in the OPIMA environment

OPIMA uses credentials to enable protected content to flow within and between compartments. Among other things, these credentials certify:

- OVMs, Applications and IPMP systems.

- Compartment identification. This implies:

- functional capabilities of the compartment, such as cryptographic support, watermarking, and system renewability, etc.

- security policies of the holder of the credential including the functional capabilities required by the other peer with which the holder wishes to establish a trusted relationship

## Required Trusted Institutions

The OPIMA specification identifies the need for institutions that manage identifiers, renewable security and revocation, compliance, etc. These may be managed by third-party private and public neutral organisations to be determined. OPIMA will not manage these infrastructure components.

The structure of these institutions, their mandate, and the format of interaction between them need to be detailed but is outside of the scope of this specification. The trustworthiness of a given compartment and the trustworthiness of content flow between compartments are enabled by the flow of credentials and their secure and predictable interpretation by the OPIMA components. The process conducted by these organisations may include auditing implementations prior to issuing credentials.

The actual number of trusted institutions will depend on how many authority functionalities a specific institution will carry out.

### Compartment ID issuance authority

Each compartment must be uniquely identified. An authority will be in charge of issuing compartment IDs.

### Credential issuance authorities

The OPIMA Credentials are issued by neutral OPIMA credential authorities, and implicitly certify the nature of the implementation of the OVM and the device on which it resides and its capabilities.

### IPMP Systems ID issuance authorities

IPMP Systems need to be uniquely identified in order for an IPMP System to be requested,  downloaded and trusted. Companies or organisations may be given the authority to issue IPMP Systems IDs within an ID assigned by one of these authorities.

### OPIMA Peer ID issuance authorities

OPIMA Peers need to be uniquely identified in order to enable secure interaction between them. Companies or organisations may be given the authority to issue OPIMA Peer IDs within an ID assigned by one of these authorities.

### Encryption, Signature and Watermarking ID issuance authorities

Encryption, Signature and Watermarking algorithms need to be uniquely identified in order for  IPMP Systems and OVMs to communicate among themselves. Only algorithms that are openly used need to be identified.

## Back-end Infrastructure

In addition to functional components on OPIMA devices, there may be an interface to back-end processing systems. These include financial, rights and usage clearinghouses. The structure of these components is proprietary; however, they shall use OVM peers to interface to the OPIMA environment.

For example, a given compartment may have a network of clearinghouses for the aforementioned operations. A peer device in this compartment will issue audit trails for transmission to the clearinghouse components. The audit trails will result in credentials if successful. The OVM at a given clearinghouse will process the information accordingly. Clearinghouse components may service multiple compartments simultaneously. The back-end infrastructure may also host the renewable security and revocation mechanisms.

## Protocols

An important part of the problem set addressed by OPIMA is the secure download of IPMP systems. In this relationship the peers play the roles of client (Peer C) and server (Peer S). An initial negotiation occurs, in which unauthenticated information is exchanged to facilitate the establishment of a Secure Authenticated Channel (SAC). This initial exchange takes place using non-OPIMA protocols. Then, the OVM establishes the OPIMA peer relationship.

Fig, 1 -OPIMA protocols

For example OPIMA peers in a Client Peer - Server Peer set up can communicate in the following way:

- An application requests the OVM to access protected content

- The OVM requests the OS to establish initial network connections

- The OPIMA Secure Authenticated Channel is established on top of this connection

- The required IPMP System is requested and downloaded by the OVM

# The OPIMA Architecture (Normative)

## OPIMA Protocols

OPIMA protocols are intended to provide a level of interoperability between OPIMA peers. The OPIMA protocols are divided into two layers:

- The first layer, called secure transport, negotiates and implements a  SAC.

- The second layer is the OPIMA common message protocol. It uses the functionality provided by the SAC layer. The messages interchanged over this protocol allow it to provide the downloading of IPMP systems services.

### First layer: Secure Authenticated Channel

This specification identifies SSL as the reference SAC protocol for inter-compartment communication. Future versions of OPIMA may provide support for a SAC protocol also usable in other environments (e.g., broadcasting, low footprint devices). The SAC establishment phase is intended to set up the encryption and decryption mechanisms used by the OVMs (e.g., select a cipher such as DES, select a chaining mode) to securely transmit IPMP Systems between the two correspondent communicating peers.

The SAC establishment phase shall also determine which keys for the aforementioned encryption and decryption mechanisms shall be used.

### Second layer: OPIMA Common Message Protocol

The OPIMA Common Message Protocol is used to establish communication between different compartments.

Such a message consists of a command and optional parameters. These parameters are listed in the tables below. The number of bits needed for these parameters are given in parentheses.

Each of the tables below includes the messages sent in sequence from “Sender” to “Recipient”.

The length field always refers to the number of bytes of the field immediately following the length field.

#### Open IPMP System download Message

| Sender | Reicipient | Content |
| --- | --- | --- |
| Peer 1 | Peer 2 | OPEN, length(8), IPMPS_ID (var) |

#### IPMP System code Messages

| Sender | Recipient | Content |
| --- | --- | --- |
| Peer 2 | Peer 1 | MSGDTA (8), length(32), message (var) |

In a push case Peer 1 will send the content and Peer 2 will receive it.

#### Close of the OPIMA download protocol

| Sender | Reiceipient | Content |
| --- | --- | --- |
| Peer 1 | Peer 2 | CLOSE |
| Peer 2 | Peer 1 | CLOSE |

Both the party sending the IPMP System and the party receiving it are allowed to send this message.

#### Message IDs

| Message Name | Value (binary) |
| --- | --- |
| MSGDTA | 00000010 |
| OPEN | 00000001 |
| CLOSE | 00000011 |

## Credential Formats

This OPIMA 1.1 specification mandates the use of X.509 v.3 certificates for credentials.

Identification of compartment and identity of OPIMA peer will be included in the certificate.

An OPIMA credential must have the X.509 v.3 format where "subject" is formatted  as a structure comprising two fields: CompartmentID and PeerID.

Therefore an OPIMA certificate is a X.509 v.3 certificate with the following relevant fields:

- Issuer: IssuerID

- Subject: SubjectID

The format of IssuerID is:

OCTET STRING(2). It contains the Certification authority ID.

The format of SubjectID is:

SEQUENCE{CompartmentID OCTET STRING(4), PeerID OCTET STRING(16)}.

## OPIMA Peer

This section describes an OPIMA peer in terms of functional units. Figure 2 depicts the relationship of the pieces described below.

Figure 2: The OPIMA Peer

### OPIMA Virtual Machine

The OPIMA peer contains a group of basic functional elements that implement the backbone of trust. This is called the OVM. The basic functionality of the OVM allows for application-specific extensions. The OVM is responsible for establishing authenticated, secure channels amongst OPIMA compliant devices.

### IPMPs

IPMPs are processes that implement Intellectual Property Management & Protection Systems (IPMP-S) inside the OPIMA peer.

### Application Services API

The OPIMA Application Services API allows services to communicate with the OVM and the IPMP system installed in the OVM. It supports the following functionalities:

- Requesting access to content

- Requesting the installation of an IPMP system component in the OVM

- Querying the installed IPMP system components

- Sending messages to the IPMP system components, among which a message to query content access rights.

The IDL definition of the Application Services API is given in 4. It comprises the functions:

- useContent

- getIpmpSystem

- queryOVM

- sendMessageToIpmp

and the following callback function:

- notifyEvent

#### useContent

This method allows services to request that content be made available by the OVM for a certain Purpose. The IPMP systems managing and protecting the content decide whether the request is acceptable.

| Input Parameters | Values |
| --- | --- |
| - Content Identification of the content to be used. | URL |
| - Content sink Identification of the sink to which content is handed off. | URL If the content sink is NULL, the sink is implied by the content.It is at the discretion of the IPMP system to decide whether this sink is acceptable. |
| - Purpose Indication of the purpose for which the content is made available. | Enumeration for this identifier is given in Table 1 (see below). It is at the discretion of the IPMP system to decide whether this purpose is acceptable. |
| - IPMPsystemID[] A set of identifiers of IPMP systems to be used to make the content available. | A set of unique identifiers of IPMP systems. Each identifier is an array of bytes containing a unique ID assigned by a registration authority. If any indicated IPMP system is not available in the OVM, the OVM may try to obtain the corresponding IPMP system component, depending on OVM policy. If the IPMP system set is empty, the OVM will decide which IPMP system to use if any, depending on the OVM policy and possibly after user interaction. |
| - Listener callback function that delivers the OVM response to the application. | Function address |
| Return Variable | Values |
| - Result | 32 bit integer, that can be either positive or negative. A positive value indicates session ID that can be used by the application to match subsequent asynchronous responses from the OVM. Negative values indicate that an error occurred and the reason of failure. |
| Asynchronous Responses | Values |
| - Success | This asynchronous response is issued at the moment that the content is available. It can be used for synchronisation purposes. |
| - Failure | This asynchronous response is issued at the moment that the content is not available or is no longer available. The reason for the failure is embedded: - Content not found at indicated URL. - Content sink not available. - Indicated IPMP system not available. - Content access disallowed by IPMP system. - IPMP specific string to be interpreted by the application. Table of values is given in the IDL definition. |

Table 1 - Actions the OVM executes on content

| IMPLICIT | The purpose is implied by the content source and the content destination |
| --- | --- |
| RENDER | Rendering the content on the OPIMA Peer |
| STOP | Stop ongoing action |
| COPY | Copy content under control of the IPMP System |
| MOVE | Move content under control of the IPMP System |
| EDIT | Edit content under control of the IPMP System |
| FORWARD | Forward content outside of the OPIMA Peer under the control of the IPMP System |
| RELEASE | Release content outside of the OPIMA Peer |
| OTHER | Anything else |

#### getIpmpSystem

This method allows services to request that an IPMP system component is installed in the OVM.

If the IPMP system is already installed in the OVM, the application is simply registered as a client of that IPMP system. Note that no compartment indication is passed by the application to the OVM, since the OVM knows its own compartment.

| Input Parameters | Values |
| --- | --- |
| IPMPsystemID Identification of the IPMP system to be installed. | An array of bytes containing a unique ID assigned by a registration authority. |
| - Source Identification of the source for the IPMP system component. | URL If Source is NULL, then the source is implied by IPMP System. |
| - Listener callback function that delivers the OVM response to the application. | Function address |
| Return Variable | Values |
| - Result | 32 bit integer, that can be either positive or negative. A positive value indicates session ID that can be used by the application to match subsequent asynchronous responses from the OVM. Negative values indicate that an error occurred and the reason of failure. |
| Asynchronous Responses | Values |
| - Success | This asynchronous response is issued at the moment that the IPMP system component has been successfully loaded. |
| - Failure | This asynchronous response is issued at the moment that the IPMP system component is not available or is no longer available. The reason for the failure is embedded: - IPMP system not present. - Source not available. - Downloading refused by source. - Not enough resources to install. Table of values is given in the IDL definition. |

#### queryOVM

This method allows applications to query the OVM. Query OVM provides the list of IPMP systems that are applicable to a certain content and gives an indication of which IPMP systems are already available within the OVM.

| Input Parameters | Values |
| --- | --- |
| - ContentId Identification of the content to be used | URL |
| - Purpose Identification of the purpose for which the content is intended to be used | Same as in “useContent” |
| - Listener callback function that delivers the OVM response to the application. | Function address |
| Return Variable | Values |
| - Result | 32 bit integer, that can be either positive or negative. A positive value indicates session ID that can be used by the application to match subsequent asynchronous responses from the OVM. Negative values indicate that an error occurred and the reason of failure. |
| Asynchronous Responses | Values |
| IpmpsID[][] Output of the function | List of alternative sets of IPMP systems. Each set is able to process the content according to the required purpose. The list also contains information of the current status of each IPMPS in the OVM (present/missing). IPMP systems are identified by IPMP system IDs, as defined above. |

#### sendMessageToIPMP

This method allows services to send messages to the IPMP systems installed in the OVM and to receive answers.

| Input Parameters | Values |
| --- | --- |
| IPMPsystemID Identification of the IPMP system to which the message is addressed. | An array of bytes containing a unique ID assigned by a registration authority. |
| - Message Type Identification of the message type | - Content Query - IPMP system proprietary - NULL message (to allow an application register itself as a receiver of messages without actually sending any message) Table of values is given in the IDL definition. |
| - Message | - URL (in case of a content query message) - Data passed to the IPMP component. |
| - Listener callback function that delivers the OVM response to the application. | Function address |
| Return Variables | Values |
| Result | 32 bit integer, that can be either positive or negative. A positive value indicates session ID that can be used by the application to match subsequent asynchronous responses from the OVM. Negative values indicate that an error occurred and the reason of failure. |
| Asynchronous Responses | Values |
| - Content query response | - Content not available. - String for display to end-user. - Data |

#### notifyEvent

This asynchronous response is issued by the OVM to the application to notify that a certain event has occurred; it can be used for synchronisation purposes.

| Input Parameters | Values |
| --- | --- |
| SessionID An identifier provided by the OVM which refers to the action to which this is a response | Same value previously returned by either UseContent, getIpmpSystem, sendMessageToIPMP or queryOVM. |
| - Status Indicates success or failure, and reasons of failure | SUCCESS if status = 0 ErrorCode if status < 0 |
| - Message An IPMP specific string to be interpreted by the application | Either - NULL (in reply to a useContent or getIpmpSystem request) or - An IPMP specific string (in reply to a sendMessageToIPMP request) or - A list of alternative sets of IPMP systems that are needed by the content in order for the OPIMA peer to perform the intended “purpose”, associated with the indication of their current status in the OVM (present/missing). IPMP systems are identified by IPMP system IDs, as defined above (in reply to a queryOVM request). |

#### Pseudo-code example for using the Application Services API

The following code gives an example of how an application might use the Application Services API in order to use content which is protected by (multiple) IPMP system(s). Note that no error messages are handled whithin this example.

function notifyEvent(sessionID, status, returnMessage);

// define the event handler. This function will be called by the

// OVM to pass information back to the application.

sessionID = queryOVM(contentURL, RENDER, &notifyEvent);

// the OVM will reply with an asynchronous event by calling

// the notifyEvent function.

// the returnMessage will contain a list of sets of IPMP systems.

// Each set is able to govern the content.

// the returnMessage will also contain a boolean flag indicating

// which IPMPS is already resident in the OVM

IpmpSet = selectIpmpsSet(returnMessage);

// this function is not defined within OPIMA. It is intended to

// select one specific IPMP system set (IpmpSet). This can involve

// user interaction or other means to select an appropriate set.

foreach(IPMPS in IpmpSet)

{

IpmpURL = lookupIpmpSystem(IPMPS);

sessionID = getIpmpSystem(IPMPS, IpmpURL, &notifyEvent);

}

// install each IPMP system of the selected set in the OVM.

// lookupIpmpSystem is not defined within OPIMA. It identifies

// the source from where the IPMPS can be obtained, if necessary.

// The OVM will reply with an asynchronous event by calling

// the notifyEvent function.

// The “status” will return an indication of success or

// failure, the returnMessage will be NULL.

sessionID = useContent(contentURL, dest, RENDER, IpmpSet, &notifyEvent);

// Renders the content from contentURL to the device indicated by dest

// using the IPMP systems from the IpmpSet.

// The OVM will reply with an asynchronous event by calling

// the notifyEvent function.

// The “status” will return an indication of success or

// failure, the returnMessage will be NULL.

### IPMP Services API

This section contains the IPMP Services API, this being the interface between the IPMP and the OPIMA Virtual Machine (OVM). The IPMP Services API is the only way of communication between the IPMP Systems on the one side and the OVM and the external world on the other side. An IDL definition of this API is provided in 4.

#### User Interface methods

The purpose of these methods is to obtain information directly from the user. This feature can be used for non-repudiation.

##### sendMessageToUser

| Input Parameters | Values |
| --- | --- |
| MessageText text to be displayed to the user. | A string of text |
| - Listener callback function that delivers the user response to the IPMP system. | Function address |
| - Return Variable | Values |
| - Status | Status > 0: The returned value is a SessionID provided by the OVM. Status < 0: The returned value is an ErrorCode; can be one of: PERMISSION_DENIED NO_RESOURCES WRONG_PARAMETERS |

##### receiveMessageFromUser

| Input Parameters | Values |
| --- | --- |
| SessionID Identifier of an action to which this is a reply | Any positive int value |
| - Response Text returned by the user (e.g. a password). | A string of text |

#### Secure Storage Interface

This set of functions allows an IPMP system to use storage capabilities provided by the OVM. Information stored upon an IPMP system request will be made available to other instantiations of the same IPMP system. The OVM enforces that an IPMP system is not allowed to access information stored by other IPMP systems. Since memory can be a scarce resource in some OVM implementations, it is not guaranteed that stored information will persist indefinitely.

##### secureStoreData

This function requests OVM to store information contained in an array of bytes. Some OVM implementations may decide to maintain storage of this information, even when the IPMP system is terminated.

| Input Parameters | Values |
| --- | --- |
| DataReference An identifier that allows different instantiations of the same IPMP system to access stored data. | - Enumeration of these identifiers is proprietary to the IPMP system. - This parameter is unique per IPMP. |
| - Data A reference to data to be stored | - This parameter is treated as an unformatted array of bytes, which are not interpreted by the OVM. |
| - Return Variable | Values |
| - Status | SUCCESS if Status = 0; Otherwise one of the following error codes: METHOD_NOT_AVAILABLE (in this OVM) INSUFFICIENT_RESOURCES PERMISSION_DENIED |

##### secureRetrieveData

This function requests OVM to retrieve information that was previously stored under DataReference by an instantiation of the calling IPMP system.

| Input Parameters | Values |
| --- | --- |
| DataReference An identifier that allows different instantiations of the same IPMP system to access stored data. | - Enumeration of these identifiers is proprietary to the IPMP system. - This parameter is unique per IPMP. |
| - Buffer Memory location, where stored data is copied into. | - This reference is provided by the IPMP system. |
| - Return Variable | Values |
| - Status | SUCCESS if Status = 0; Otherwise one of the following error codes: METHOD_NOT_AVAILABLE (in this OVM) INSUFFICIENT_RESOURCES (buffer is too small) NO_DATA PERMISSION_DENIED |

##### secureDeleteData

This function requests OVM to destroy information that was previously stored under DataReference by an instantiation of the calling IPMP system.

| Input Parameters | Values |
| --- | --- |
| DataReference An identifier that allows different instantiations of the same IPMP system to access stored data. | - Enumeration of these identifiers is proprietary to the IPMP system. - This parameter is unique per IPMP. |
| - Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

#### Encryption and Decryption Engines

The IPMP Systems can access standard cryptographic algorithms (implemented in hardware or software) through the following functions.

##### queryEncryptionAlgorithms

| Return Variables | Values |
| --- | --- |
| AlgorithmList | Array of strings indicating the supported encryption and decryption algorithms. |

##### encrypt

This method encrypts a set of bytes. It is intended primarily to encrypt management messages.

| Input Parameters | Values |
| --- | --- |
| - Algorithm Identification of the algorithm used in the encryption. | A text string |
| - Params The parameters of the algorithm | An array of bytes |
| - Key The key used in the encryption. | An array of bytes |
| Input/Output Parameters | Values |
| - Data. Input: the data to be encrypted Output: the encrypted data | An array of bytes |
| Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

##### initEncryption

This method initialises content encryption. The implementation of this method is able to access content on the basis of the clearContentId parameter. The return variable is a reference to the encrypted content or (if negative) an error code.

| Input Parameters | Values |
| --- | --- |
| - Algorithm Identification of the algorithm used in the encryption. | A text string |
| - Params The parameters of the algorithm | An array of bytes |
| - ClearContentId Reference to the content stream that must be encrypted | Integer |
| Return Variable | Values |
| - EncryptedContentId | a reference to the encrypted content or (if negative) an error code. |

##### updateEncryptionKeys

This method starts or continues content encryption using new or modified keys. An array of keys is used to allow for key changes.

| Input Parameters | Values |
| --- | --- |
| - Keys The keys used in the encryption. | An array of keys (that is, an array of array of bytes). |
| - ClearContentId Reference to the content stream that must be encrypted | Integer |
| Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

##### stopEncryption

| Input Parameters | Values |
| --- | --- |
| - ClearContentId Reference to the content stream | Integer |
| Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

##### decrypt

This method decrypts a set of bytes. It is intended primarily to decrypt management messages.

| Input Parameters | Values |
| --- | --- |
| - Algorithm Identification of the algorithm used in the decryption. | A text string |
| - Params The parameters of the algorithm | An array of bytes |
| - Key The key used in the decryption. | An array of bytes |
| Input/Output Parameters | Values |
| - Data. Input: the data to be decrypted Output: the decrypted data | An array of bytes |
| Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

##### initDecryption

This method initialises content decryption. The implementation of this method is able to access content on the basis of the EncryptedContentId parameter. The return variable is a reference to the decrypted content or (if negative) an error code.

| Input Parameters | Values |
| --- | --- |
| - Algorithm Identification of the algorithm used in the decryption. | A text string |
| - Params The parameters of the algorithm | An array of bytes |
| - EncryptedContentId Reference to the content stream that must be decrypted | Integer |
| Return Variable | Values |
| - DecryptedContentId | a reference to the decrypted content or (if negative) an error code. |

##### updateDecryptionKeys

This method starts or continues content decryption using new or modified keys. An array of keys is used to allow for key changes.

| Input Parameters | Values |
| --- | --- |
| - Keys the keys used in the decryption. | An array of keys (that is, an array of array of bytes). |
| - EncryptedContentId Reference to the content stream that must be decrypted | Integer |
| Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

##### stopDecryption

| Input Parameters | Values |
| --- | --- |
| - EncryptedContentId Reference to the content stream | Integer |
| Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

#### Signature Engines

This interface is used by IPMP Systems that wish to use public key cryptography in order to perform digital signature processing

##### querySignatureAlgorithms

| Return Variables | Values |
| --- | --- |
| AlgorithmList | Array of strings indicating the supported digital signature algorithms. |

A Registration Authority will issue standard identifiers for established algorithms.

##### verifySignature

This method verifies a digital signature. It is intended primarily to verify the authenticity of management messages.

| Input Parameters | Values |
| --- | --- |
| - Algorithm The algorithm used in the verification. | A text string |
| - Params The parameters of the algorithm | An array of bytes |
| - PublicKey The public key used in the verification. | An array of bytes |
| - Data The data that has been signed. | An array of bytes |
| - Signature The digital signature to be verified | An array of bytes |
| Return Variable | Values |
| - Status | TRUE if Status = 1 FALSE if Status = 0 ErrorCode if Status < 0 |

##### verifyContentSignature

This method verifies a digital signature. It is intended to verify the authenticity of content. The implementation of this method is able to access content on the basis of the contentId parameter.

| Input Parameters | Values |
| --- | --- |
| - Algorithm The algorithm used in the verification. | A text string |
| - Params The parameters of the algorithm | An array of bytes |
| - PublicKey The public key used in the verification. | An array of bytes |
| - ContentId reference to the content that has been signed. | An integer value |
| - Signature The digital signature to be verified | An array of bytes |
| Return Variable | Values |
| - Status | TRUE if Status = 1 FALSE if Status = 0 ErrorCode if Status < 0 |

##### generateSignature

This method generates a digital signature. It is intended primarily to sign management messages.

| Input Parameters | Values |
| --- | --- |
| - Algorithm The algorithm used in the verification. | A text string |
| - Params The parameters of the algorithm | An array of bytes |
| - PrivateKey The private key used in the generation. | An array of bytes |
| - Data The data to be signed. | An array of bytes |
| Output Parameters | Values |
| - Signature The resulting digital signature | An array of bytes |
| Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

##### generateContentSignature

This method generates a digital signature. It is intended to sign content. The implementation of this method is able to access content on the basis of the contentId parameter.

| Input Parameters | Values |
| --- | --- |
| - Algorithm The algorithm used in the verification. | A text string |
| - Params The parameters of the algorithm | An array of bytes |
| - PrivateKey The private key used in the generation.. | An array of bytes |
| - ContentId reference to the content that has been signed. | An integer value |
| Output Parameters | Values |
| - Signature The resulting digital signature | An array of bytes |
| Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

#### Watermarking Engines

In  case the OVM provides access to watermarking engines the following  interfaces are used.

##### queryWatermarkAlgorithms

| Return Variables | Values |
| --- | --- |
| AlgorithmList | Array of strings indicating the supported watermarking algorithms. |

A Registration Authority will issue standard identifiers for established algorithms.

##### extractWatermark

This method starts watermark extraction from content identified by contentId. If the content is a stream, watermark extraction can be stopped by calling stopWatermarkExtraction. A call back interface is used to transfer the watermark.

| Input Parameters | Values |
| --- | --- |
| - Algorithm The watermarking algorithm. | A text string |
| - Params The parameters of the algorithm | An array of bytes |
| - ContentId Reference to the watermarked content | An integer value |
| Listener Callback function implemented by the IPMP System to accept the asynchronous responses that would be issued by the OVM whenever watermark is detected | An identifier of a function |
| Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

##### stopWatermarkExtraction

This method stops watermark extraction from a content stream identified by contentId.

| Input Parameters | Values |
| --- | --- |
| - ContentId Reference to the content stream from which the a watermark is extracted. | An integer value |
| Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

##### newWatermark

This is the call back interface for the watermark engine. It should be implemented by IPMP components that call the watermark engine.

| Input Parameters | Values |
| --- | --- |
| - ContentId Reference to the watermarked content | An integer value |
| - Watermark Value of the extracted watermark | An array of bytes |

##### insertWatermark

This method starts watermark insertion into content identified by sourceContentId. The return variable is the sinkContentId. If the content is a stream, watermark insertion can be stopped by calling stopWatermarkInsertion.

| Input Parameters | Values |
| --- | --- |
| - Algorithm The watermarking algorithm. | A text string |
| - Params The parameters of the algorithm | An array of bytes |
| - SourceContentId Reference to the content to be watermarked. | An integer value |
| Return Variable | Values |
| - SinkContentId | a reference to the watermarked content or (if negative) an error code. |

##### stopWatermarkInsertion

This method stops watermark extraction from a content stream identified by contentId.

| Input Parameters | Values |
| --- | --- |
| - SourceContentId Reference to the content to be watermarked. | An integer value |
| Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

#### Smart Cards

Smart cards accessible through local smart card readers can be accessed by means of this API.

The following two structures represent messages in ISO 7816 format:

struct CommandAPDU

{

byte cla;

byte ins;

byte P1;

byte P2;

int lc;			//      The length of the data field of the APDU.

int le;			//      The expected length of the ResponseAPDU.

byte[] data;

}

struct ResponseAPDU

{

byte sw1

byte sw2

byte[] data

}

##### addCTListener

This method adds a Card Terminal listener to receive an indication of card insertion or removal from this card terminal.

| Input Parameters | Values |
| --- | --- |
| - Listener The listener to add | The function addresses of cardInserted and cardRemoved |
| Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

##### removeCTListener

Removes a CTListener so it no longer receives CardTerminalEvents from this card terminal

| Input Parameters | Values |
| --- | --- |
| - Listener The CTListener to remove. | The function address of either cardInserted or cardRemoved |
| Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

##### getSlotId

Returns the list of slots belonging to this Card Terminal.

| Output Parameters | Values |
| --- | --- |
| - SlotId The array of slot identifiers. | An array of int |
| Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

##### isCardPresent

Checks whether there is a smart card present in a particular slot

| Input Parameters | Values |
| --- | --- |
| - SlotId The slot to check for a card | A positive integer value |
| Return Variable | Values |
| - Status Indicates if there is a smart card inserted in the slot. | TRUE if Status = 1 FALSE if Status = 0 ErrorCode if Status < 0 |

##### openSlotChannel

Opens a SlotChannel on Slot number slotID.

| Input Parameters | Values |
| --- | --- |
| - SlotId The number of the slot for which a SlotChannel is requested. | A positive integer value |
| Return Variable | Values |
| - Status Identification of the session to the smart card or error code. | SlotSessionId if Status > 0 ErrorCode if Status < 0 |

##### closeSlotChannel

Closes a SlotChannel identified by slotSessionId.

| Input Parameters | Values |
| --- | --- |
| - SlotSessionId The smart card session to close. | A positive integer value |
| Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

##### getATR

Returns the answer-to-reset (ATR) response of the card inserted in slot slotID.

| Input Parameters | Values |
| --- | --- |
| - SlotId The slot identifier | A positive integer value |
| - Ms A timeout in milliseconds. | A positive integer value; zero is interpreted as “no timeout” |
| Output Parameters | Values |
| - ATR The ATR response in form of a byte array. | An array of bytes |
| Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

##### reset

Resets a smart card inserted in a slot.

| Input Parameters | Values |
| --- | --- |
| - SlotSessionId The open slot channel attached to the slot. | A positive integer value |
| - Timeout A timeout in milliseconds. | A positive integer value; zero is interpreted as “no timeout” |
| Output Parameters | Values |
| - ATR The ATR response in form of a byte array. | An array of bytes |
| Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

##### sendAPDU

Sends a command to the smart card and waits for the response.

| Input Parameters | Values |
| --- | --- |
| - SlotSessionId The open slot channel attached to the slot. | A positive integer value |
| - CommandAPDU The CommandAPDU to send. | See the “CommandAPDU” structure defined above. |
| - Timeout A timeout in milliseconds. | A positive integer value; zero is interpreted as “no timeout” |
| Output Parameters | Values |
| - ResponseAPDU The ResponseAPDU as received from the card. | See the “ResponseAPDU” structure defined above. |
| Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

The following call backs are to be implemented by the IPMP system:

##### cardInserted

Notifies that a smart card has been inserted in a slot identified by slotId.

| Input Parameters | Values |
| --- | --- |
| - SlotId The slot identifier. | A positive integer value |

##### cardRemoved

Notifies that a smart card has been removed from a slot identified by slotId.

| Input Parameters | Values |
| --- | --- |
| - SlotId The slot identifier. | A positive integer value |

#### Abstract Access to Content

##### installCallbackContentAccess

This function is a request from the IPMP system to the OVM, to install a callback function for this set of methods.

| Input Parameters | Values |
| --- | --- |
| Listener Callback function implemented by the IPMP system to accept an asynchronous response issued by the OVM. | An identifier of a function. |
| - Return Variable | Values |
| Status | SUCCESS (if Status = 0); Otherwise one of the following error codes: INSUFFICIENT_RESOURCES PERMISSION_DENIED |

##### abstractContentAccess

This is a previously installed callback function, through which the OVM calls the IPMP system for a specific purpose. Usually this function should be called by the OVM upon request from the Application (see useContent function), thus implementing an indirect call of the IPMP system from an Application.

| Input Parameters | Values |
| --- | --- |
| ContentId An identifier of the Content. | The result of the translation of the URL that has been passed from the Application (see useContent function). |
| - Content sink Identification of the sink to which content is handed off. | URL If the content sink is NULL, the sink is implied by the content. It is at the discretion of the IPMP system to decide whether this sink is acceptable. |
| - Purpose An identifier of the purpose to access content. | - Enumeration for this identifier is given in Table 1 (see 3.3.3.1). - The OPIMA definition for this enumeration includes a set of purposes, as well as the possibility to accommodate proprietary values. |
| - SessionId An identifier of the session generated by the OVM | - This value is generated by the OVM and passed as input to the IPMP system. - It will be used by replyToContentAccess to identify the session. |
| - Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

##### replyToContentAccess

This function implements the response from the IPMP System to abstractContentAccess. This will be translated by the OVM into a call of the “notifyEvent” callback function, which is part of the Application Services API.

| Input Parameters | Values |
| --- | --- |
| SessionId An identifier of the session. | This value has been received from the OVM through the call to abstractContentAccess. |
| - Status A status indication for the application | SUCCESS (Status = 0) ErrorCode (Status < 0) |
| - Message A reference containing the requested action. | An array of bytes |
| - Return Variable | Values |
| - Status A status indication for the IPMP system | SUCCESS if Status = 0 ErrorCode if Status < 0 |

#### Abstract Access to Rules

In the cases where the content is not directly accessible to an IPMP System in order to be processed, the rules may be accessed using this interface.

##### obtainUserRules

This function retrieves the rules associated with a user.

| Input Parameters | Values |
| --- | --- |
| UserId An identifier of the user. | This parameter is treated as an unformatted array of bytes. |
| - Listener An identifier of a function. | Callback function implemented by the IPMP system to accept the asynchronous response that will be issued by the OVM. |
| - Return Variable | Values |
| - Status | Status > 0: The returned value is a sessionId provided by the OVM to be used by the IPMP system to manage the asynchronous response. This implies that the call has been successfully accepted by the OVM. Status < 0: METHOD_NOT_AVAILABLE (in this OVM implementation) INSUFFICIENT_RESOURCES PERMISSION_DENIED Invalid_parameter |

##### obtainContentRules

This function retrieves the rules associated with a content.

| Input Parameters | Values |
| --- | --- |
| sourceContentId An identifier of the content. | This parameter has been passed to the IPMP system in the abstractContentAccess function. |
| - listener An identifier of a function. | Callback function implemented by the IPMP system to accept the asynchronous response that will be issued by the OVM. |
| - Return Variable | Values |
| - Status | Status > 0: The returned value is a sessionId provided by the OVM to be used by the IPMP system to manage the asynchronous response. This implies that the call has been successfully accepted by the OVM. Status < 0: METHOD_NOT_AVAILABLE INSUFFICIENT_RESOURCES PERMISSION_DENIED Invalid_parameter |

##### newRules

This is a callback function implemented by the IPMP system. The intended use of this function is a listener for either obtainContentRules or obtainUserRules. The SessionId is used to associate a request to a response.

| Input Parameters | Values |
| --- | --- |
| SessionId An identifier of the asynchronous session between the IPMP system and the OVM. | This parameter has been received as a return value from obtainUserRules or obtainContentRules. |
| - Buffer An identifier of the rules requested. | Requested rules. Buffer will be reused by the OVM. |

Note: To prevent memory leakage in the OVM, content in the Buffer parameter should be copied into the IPMP system memory space before returning from this callback function.

##### updateContentRules

This function updates the rules associated with a content.

| Input Parameters | Values |
| --- | --- |
| SinkContentId An identifier of the sink for the content. | Any integer value |
| - Buffer An identifier of a function. | Buffer contains the update rule that must be forwarded to the Content sink. |
| - Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

#### Abstract Access to OPIMA Peers

The purpose of this interface is to exchange IPMP system proprietary management and control messages between OPIMA peers. This interface can also be used for communication between local IPMP Systems.

##### openConnection

This method opens a connection to an OPIMA peer.

| Input Parameters | Values |
| --- | --- |
| - Peer URL of the destined OPIMA peer. | An URL (a special case is “localhost”) |
| - IPMPsystemID Identification of the destined IPMP system. | An array of bytes containing a unique ID assigned by a registration authority. |
| - ConnectionSpec specification of the connection to be opened. | The connection specification ID specifies which type of connection to use - ID=0 cleartext - ID=1 OPIMA SAC - ID > 1 compartment specific |
| Listener A function which listens to messages sent by the other peer | Function reference |
| Return Variable | Values |
| - Status Either an identification of the connection or an error code | ConnectionId if Status > 0 ErrorCode if Status < 0 |

##### closeConnection

This method closes a connection to an OPIMA peer.

| Input Parameters | Values |
| --- | --- |
| - ConnectionId Identification of the connection as returned earlier by the openConnection method. | A positive integer value. |
| Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

##### addConnectionListener

This method installs a listener for new connections that other OPIMA peers try to establish.

| Input Parameters | Values |
| --- | --- |
| - Peer URL of the originating OPIMA peer. If NULL, all OPIMA peers can establish a connection. | An URL |
| - IPMPsystemID Identification of the originating IPMP system. | An array of bytes containing a unique ID assigned by a registration authority. |
| Listener A function which listens to connection requests from other OPIMA Peers | Function reference |
| Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

##### sendMessage

This method sends a message on the designated connection.

| Input Parameters | Values |
| --- | --- |
| - ConnectionId identification of the connection as returned earlier by the openConnection method. | A positive integer value. |
| - Message Message to send. | An array of bytes. |
| Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

The following call backs are to be implemented by the IPMP system.

##### newConnection

This method signals the establishment of a new connection.

| Input Parameters | Values |
| --- | --- |
| - Peer URL of the originating OPIMA peer. | An URL |
| - IPMPsystemID Identification of the destined IPMP system. | An array of bytes containing a unique ID assigned by a registration authority. |
| - ConnectionId Identification of the connection. | A positive integer value. |
| Return Variable | Values |
| - Status Status indication provided by the IPMP system to the OVM. | SUCCESS if Status = 0 ErrorCode if Status < 0 |

##### receiveMessageFromPeer

This method receives a message on the designated connection.

| Input Parameters | Values |
| --- | --- |
| - ConnectionId Identification of the connection as returned earlier by the openConnection method. | A positive integer value. |
| - Message The received message. | An array of bytes. |
| Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

#### Abstract Access to Applications

Applications may access the local IPMP Systems if the IPMP Systems have previously installed the appropriate callback via this interface.

##### installCallbackApplication

This is an IPMP System request to the OVM to install a callback function for the application.

| Input Parameters | Values |
| --- | --- |
| - Listener The listener of messages. | The function address of receiveMessageFromApplication |
| Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

##### replyMessage

The IPMP system replies to a message received by means of a receiveMessageFromApplication callback with this method.

| Input Parameters | Values |
| --- | --- |
| SessionId An identifier received in a receiveMessageFromApplication to which this is a response. | A positive integer value |
| - Status Indicates success or failure, and reasons of failure. | SUCCESS if Status = 0 ErrorCode if Status < 0 |
| - Message An IPMP specific string to be interpreted by the application. | Either NULL or an IPMP specific string |
| Return Variable | Values |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

The following callback is to be implemented by the IPMP system:

##### receiveMessageFromApplication

Callback function by which the application can access the IPMP system (via the OVM). It is the OVM which calls the IPMP system by means of this method.

| Input Parameters | Values |
| --- | --- |
| - SessionId An identifier provided by the OVM which refers to this action. | A positive integer value. |
| - MessageType An IPMP specific string to be interpreted by the IPMP. | An enumeration type with three values: CONTENT_QUERY, PROPRIETARY_MESSAGE, NULL_MESSAGE. |
| - Message An IPMP specific string to be interpreted by the IPMP. | Either NULL or an IPMP specific string. |

#### Life-cycle Control

This interface is used to control the execution state of the IPMP System. The following functions are to be implemented by IPMP systems for the purpose of life cycle control:

##### initialize

This method is called immediately after installation of the IPMP system. It allows the IPMP system to take care of any necessary installation procedures.

| Return Variable | Values |
| --- | --- |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

##### terminate

This method is called before de-installation of the IPMP system. It allows the IPMP system to take care of all necessary termination procedures. After this method  has returned, the IPMP system will be removed from the OVM.

| Return Variable | Values |
| --- | --- |
| - Status | SUCCESS if Status = 0 ErrorCode if Status < 0 |

The following functions are to be implemented by the OVM for the purpose of life cycle control:

##### remove

This method removes the IPMP system upon its request. It never returns.

##### update

This method removes the current instance of the IPMP system and requests downloading of a new instance. It never returns.

| Input Parameters | Values |
| --- | --- |
| - Source Identification of the source for the new IPMP system. | A URL. |

#### Locale interface

This interface lets an IPMP system component running on top of an OVM know in a secure way where and when it is running.

##### getTime

This method returns either an error number (if negative) or the number of seconds since "the Epoch" (the time 0 hours, 0 minutes, 0 seconds, January 1, 1970 Coordinated Universal Time - UTC). This introduces a discontinuity (a Y2K-like problem) in year 2038 that must be handled by the IPMPS component making use of this function.

| Return variable | Values |
| --- | --- |
| - Status | Time information if Status >= 0 ErrorCode if Status < 0 |

##### getCountry

This method returns either an empty string or an uppercase ISO 3166 2-letter code representing the country for which the OVM has been configured.

| Return variable | Values |
| --- | --- |
| - Country | NULL or an uppercase ISO 3166 2-letter country code |

##### getLanguage

This method returns either an empty string or the language code for this OVM, which will be a lowercase ISO 639 code.

| Return variable | Values |
| --- | --- |
| - Language | NULL or a lowercase ISO 639 language code |

# IDL definition of the OPIMA APIs (Normative)

/*

*  IDL definition of the OPIMA interfaces.

*  Note: the OPIMA peer is composed by Applications, IPMP systems, and the OPIMA VM.

*  The Applications implement: ApplicationServicesListener, ErrorCodes.

*  the IPMP systems implement: all the IPMP Listeners interfaces, ErrorCodes.

*  the OPIMA VM implements: ApplicationServices, all the IPMP Services interfaces, ErrorCodes.

*/

module OPIMA {

/*--- return values (positive values); these can be either Boolean values or Session IDs ---*/

interface BooleanResult {

const long IS_TRUE = 1;

const long IS_FALSE = 0;

};

interface SessionResult {

const long SUCCESS = 0;

};

/*--- error codes (negative values). These can be extended for compartment specific errors ---*/

interface ErrorCodes {

const long CONTENT_NOT_FOUND = -1;		// Content not found at the indicated URL.

const long SINK_NOT_AVAILABLE = -2;		// Content sink not available.

const long CONTENT_ACCESS_NOT_ALLOWED = -3;	// Content access prohibited by IPMP System

const long UNKNOWN_SOURCE = -4;			// Source could not be reached / source unknown

const long BUSY_SOURCE = -5;			// Source not available at this moment / source busy

const long INVALID_SOURCE = -6;			// Source was not authenticated as a valid source for the requested IPMP system.

const long IPMPS_NOT_AVAILABLE = -7;		// Requested IPMP system is not available at the source.

const long DOWNLOAD_NOT_ALLOWED = -8;		// Source refuses to download the IPMP system to this OPIMA peer.

const long INSUFFICIENT_RESOURCES = -9;		// Insufficient resources to perform the requested action.

const long TIMEOUT_EXPIRED = -10;		// A timeout expired before completion of the task.

const long PERMISSION_DENIED = -11;		// The requested action is not allowed.

const long WRONG_PARAMETER_1 = -12;		// Error in first parameter

const long WRONG_PARAMETER_2 = -13;		// Error in second parameter

const long WRONG_PARAMETER_3 = -14;		// Error in third parameter

const long WRONG_PARAMETER_4 = -15;		// Error in fourth parameter

const long WRONG_PARAMETER_5 = -16;		// Error in fifth parameter

const long METHOD_NOT_AVAILABLE = -17;		// The requested function has not been implemented on this OVM

const long CARD_NOT_PRESENT = -18;		// The smart card has been removed

const long COMMUNICATION_ERROR = -19;		// Transmission error between the terminal and the smart card

const long RULES_NOT_AVAILABLE = -20;		// The OVM was not able to extract rules from content

const long CANNOT_UPDATE_RULES = -21;		// The OVM was not able/allowed to modify rules

const long APPLICATION_TERMINATED = -22;	// The application was terminated

};

/*--- complex types used as method parameters in more than one interface ---*/

typedef string URL;

typedef sequence <octet, 256000> Buffer;

typedef sequence <string, 1024> List;

typedef sequence <octet, 255> IPMPSid;

enum Purpose {IMPLICIT, RENDER, STOP, COPY, MOVE, EDIT, FORWARD, RELEASE, OTHER};

enum MessageType {CONTENT_QUERY, NULL_MESSAGE, PROPRIETARY_FORMAT};

/*-------- listeners defined in Application Services ---------*/

interface ApplicationServicesListener {

struct IPMPSidContainer {

IPMPSid id;

boolean exists;

} ;

typedef sequence<sequence<IPMPSidContainer,256>, 256> IPMPSidContainerList;

enum Selector {IPMP_MESSAGE, OVM_MESSAGE };

union replyData switch ( Selector )

{

case IPMP_MESSAGE: Buffer IpmpMessage;

case OVM_MESSAGE: IPMPSidContainerList OvmMessage;

};

void notifyEvent(in long sessionId, in long status, in replyData message);

};

/*-------- Application Services ---------*/

interface ApplicationServices {

typedef sequence<IPMPSid,256> IpmpSet;

long useContent(in URL contentSource, in URL contentSink, in Purpose purpose, in IpmpSet set, in ApplicationServicesListener listener);

long queryOVM(in URL contentId, in Purpose p, in ApplicationServicesListener listener);

long getIpmpSystem(in IPMPSid id, in URL source, in ApplicationServicesListener listener );

long sendMessageToIPMP(in IPMPSid id, in MessageType mt, in Buffer message, in ApplicationServicesListener listener);

};

/*-------- listeners defined in IPMP Services ----------*/

interface UI_Listener {

void receiveMessageFromUser(in long sessionId, in string response);	// user interface

};

interface WE_Listener {

void newWatermark(in long contentId, in Buffer watermark);	// watermark engine

};

interface SC_Listener {

void cardInserted(in long slotId);		// smart card

void cardRemoved(in long slotId);		// smart card

};

interface AAC_Listener {

long abstractContentAccess( in long contentSourceId, in URL contentSink, in Purpose purpose, in long sessionId);	// abstract access to content

};

interface AAR_Listener {

void newRules(in long sessionId, in Buffer buffer);		// abstract access to rules

};

interface AAP_Listener {

long newConnection(in URL peer, in IPMPSid id, in long connectionId);		// abstract access to Peers (SAC)

long receiveMessageFromPeer(in long connectionId, in Buffer message);		// abstract access to Peers (SAC)

};

interface AAA_Listener {

void receiveMessageFromApplication(in long sessionID, in MessageType mt, in Buffer message);	// abstract access to application

};

/* this is the only mandatory interface that must be implemented in an IPMPS component */

interface LCC_Listener {

long initialize();			// life-cycle control

long terminate();			// life-cycle control

};

/*--------- IPMP Services ----------*/

interface UserInterface {

long sendMessageToUser(in string messageText, in UI_Listener listener);

};

interface SecureStorage {

long secureStoreData (in long dataReference, in Buffer buffer);

long secureRetrieveData(in long dataReference, out Buffer buffer);

long secureDeleteData (in long dataReference);

};

interface EncryptionEngine {

typedef sequence <Buffer, 1024> KeyList;

List queryEncryptionAlgorithms();

long encrypt(in string algorithm, in Buffer params, in Buffer key, inout Buffer data);

long initEncryption(in string algorithm, in Buffer params, in long clearContentId);

long updateEncryptionKeys(in KeyList keys, in long clearContentId);

long stopEncryption(in long clearContentId);

long decrypt(in string algorithm, in Buffer params, in Buffer key, inout Buffer data);

long initDecryption(in string algorithm, in Buffer params, in long encryptedContentId);

long updateDecryptionKeys(in KeyList keys, in long encryptedContentId);

long stopDecryption(in long encryptedContentId);

};

interface SignatureEngine {

List querySignatureAlgorithms();

long verifySignature(in string algorithm, in Buffer params, in Buffer publicKey, in Buffer data, in Buffer signature);

long verifyContentSignature(in string algorithm, in Buffer params, in Buffer publicKey, in long contentId, in Buffer signature);

long generateSignature(in string algorithm, in Buffer params, in Buffer privateKey, in Buffer data, out Buffer signature);

long generateContentSignature(in string algorithm, in Buffer params, in Buffer privateKey, in long contentId, out Buffer signature);

};

interface WatermarkEngine {

List queryWatermarkAlgorithms();

long extractWatermark(in string algorithm, in Buffer params, in long contentId, in WE_Listener listener);

long stopWatermarkExtraction(in long contentId);

long insertWatermark(in string algorithm, in Buffer params, in long sourceContentId);

long stopWatermarkInsertion(in long sourceContentId);

};

interface SmartCard {

typedef sequence <long,   1024> SlotIdList;

/* these structures declare Application Protocol Data Units as defined in ISO 7816 */

struct CommandAPDU {

octet cla;

octet ins;

octet p1;

octet p2;

short lc;

short le;

sequence <octet, 65535> data;

};

struct ResponseAPDU {

octet sw1;

octet sw2;

sequence <octet, 65535> data;

};

long addCTListener(in SC_Listener listener);

long removeCTListener(in SC_Listener listener);

long getSlotId(out SlotIdList slotId);

long isCardPresent(in long slotId);

long openSlotChannel(in long slotId);

long closeSlotChannel(in long slotSessionId);

long getATR(in long slotId, in long ms, out Buffer ATR);

long reset(in long slotSessionId, in long ms, out Buffer ATR);

long sendAPDU(in long slotSessionId, in CommandAPDU capdu, in long ms, out ResponseAPDU rapdu);

};

interface AbstractAccessToContent {

long installCallBackContentAccess(in AAC_Listener listener);

long replyToContentAccess(in long sessionId, in long status, in Buffer message);

};

interface AbstractAccessToRules {

long obtainUserRules(in Buffer userId, in AAR_Listener listener);

long obtainContentRules(in long sourceContentId, in AAR_Listener listener);

long updateContentRules(in long sinkContentID, in Buffer buffer);

};

interface AbstractAccessToPeers {

long openConnection(in URL peer, in IPMPSid id, in long connectionSpecId, in AAP_Listener listener);

long closeConnection(in long connectionId);

long addConnectionListener(in URL peer, in IPMPSid id, in AAP_Listener listener);

long sendMessage(in long connectionId, in Buffer message);

};

interface AbstractAccessToApplications {

long installCallBackApplication(in AAA_Listener listener);

long replyMessage(in long sessionId, in long status, in Buffer message);

};

interface LifeCycleControl {

void remove();

void update(in URL source);

};

interface Locale {

long getTime();

string getCountry();

string getLanguage();

};

};

# Annexes (Informative)

## Acronyms

API	Application Programming Interface

DVB	Digital Video Broadcasting

ID	Identity

IPMP	Intellectual Property Management & Protection

MPEG	Moving Picture Experts Group

OVM	OPIMA Virtual Machine

SAC	Secure Authenticated Channel

TS	Transport Stream (i.e. MPEG-2 Transport Stream)

URL	Uniform Resource Locator

## Definitions

| Compartment | A class of OPIMA enabled devices that share some common elements in their IPMP interfaces and/or architectural components. |
| --- | --- |
| Content | All digital data representing audio, graphics, text, video and associated metadata |
| IPMP System | The system which protects and manages intellectual property rights associated with content. |
| Credentials | A set of authenticated identifiers, certifying the compartment ID and the peer ID; |
| OVM | A group of basic functional elements that implement a secure execution environment for IPMP Systems |
| OPIMA Peer | An implementation of the OPIMA specification running on a device |
| Protected Content | Content protected by an IPMP System with associated Rules. |
| Rules | Statements that govern the way a specific piece of content protected by an IPMP System can be managed |